To achieve such thermal segmentation of slices into categories like Core, Shells, Overhangs, and Hatches in libslic3r, you will need to extend the slicing pipeline and modify the data model to support these detailed segment types. You’re essentially transforming libslic3r from an extrusion slicer into a thermal-aware vector generator for SLM, so this requires some structural changes.

## 🧩 Step-by-Step Guide to Add Thermal Segment Types

### ✅ Step 1: Define Your Segment Types in an Enum

Create a new enum similar to what you’ve written in your post, e.g.,:

cpp

CopyEdit

enum ThermalSegmentType {

CoreContour_Volume           = 1,

CoreContour_Overhang         = 2,

HollowShell1Contour_Volume   = 3,

HollowShell1Contour_Overhang = 4,

HollowShell2Contour_Volume   = 5,

HollowShell2Contour_Overhang = 6,

CoreOverhangHatch            = 7,

CoreNormalHatch              = 8,

CoreContourHatch             = 9,

HollowShell1OverhangHatch    = 10,

HollowShell1NormalHatch      = 11,

HollowShell1ContourHatch     = 12,

HollowShell2OverhangHatch    = 13,

HollowShell2NormalHatch      = 14,

HollowShell2ContourHatch     = 15,

SupportContourVolume         = 16,

SupportHatch                 = 17,

PointSequence                = 18,

ExternalSupports             = 19,

CoreContourHatchOverhang     = 20,

HollowShell1ContourHatchOverhang = 21,

HollowShell2ContourHatchOverhang = 22

};

Put this enum in a shared header file, such as ThermalSegmentType.hpp.

### ✅ Step 2: Extend ExtrusionEntity or Create a New Class

Since you're not doing extrusion, you can:

- Create a new class, e.g., ScanSegment, which stores a path and a ThermalSegmentType.

cpp

CopyEdit

struct ScanSegment {

Polylines geometry;

ThermalSegmentType type;

};

You can also subclass ExtrusionEntityCollection if you prefer to reuse existing infrastructure.

### ✅ Step 3: Modify LayerRegion to Store Segments

In LayerRegion (or a new custom class you derive from it), store segmented data:

cpp

CopyEdit

std::vector<ScanSegment> scan_segments;

Modify or override the method:

- LayerRegion::fill_surfaces() → replace standard Fill* calls with logic that determines the thermal segment and fills scan_segments.

For example:

cpp

CopyEdit

if (region_is_core && is_on_volume)

scan_segments.push_back({generated_paths, CoreNormalHatch});

else if (region_is_shell1 && is_on_overhang)

scan_segments.push_back({generated_paths, HollowShell1OverhangHatch});

### ✅ Step 4: Extend PrintLayer or PrintObject to Organize by Layer

You’ll need to collect all ScanSegments from all LayerRegions in a given layer. Modify PrintLayer or a new container class:

cpp

CopyEdit

std::vector<ScanSegment> all_segments;

Loop through all LayerRegions and aggregate them.

### ✅ Step 5: Bypass G-code and Export These Segments

Instead of writing G-code, write a custom exporter:

cpp

CopyEdit

class SLMExporter {

public:

void export_layer(const std::vector<ScanSegment> &segments, const std::string &filename);

};

Output could be in CSV, JSON, XML, or a binary format understood by your SLM printer control software.

### ✅ Step 6: (Optional) Geometry-Based Segmentation Logic

Add logic to decide whether a region is:

- Overhang: using overhang detection (check face normal vs. build direction)

- Shell Level: based on distance from outer surface

- Support: intersecting with support volumes

You might add these in a ThermalClassifier class:

cpp

CopyEdit

class ThermalClassifier {

public:

ThermalSegmentType classify(const Surface &s, const RegionType &region);

};

## 🔧 Where to Hook in the Codebase

| Location | Action |
| --- | --- |
| src/LayerRegion.cpp | Override fill_surfaces() to generate ScanSegments instead of ExtrusionPaths |
| src/PrintLayer.cpp | Collect segments across regions |
| src/PrintObject.cpp | Possibly assign types based on part geometry |
| src/exporters/SLMExporter.cpp | Write your custom path format |
| (New) ThermalSegmentType.hpp | Define all your types cleanly |
| (Optional) src/ThermalClassifier.cpp | Add rules for overhang, shell classification |

## 🔄 Optional: Visual Debugging

- You can render the segment types using different colors in the GUI.

- Add segment type labels in SVG outputs (svg_export_layer) for inspection.

Would you like a sample C++ struct and method to generate a CoreNormalHatch scanline fill from a polygon?

4o

Top of Form

Search

Reason

Deep research

Create image

Bottom of Form